Provisional Patent Application — Road-Wagon Transit System v1.0.docx

Drafted as Indian Form-2 provisional, worded to support PCT conversion within the 12-month priority window without rework. 13 sections including the investor-grade structure you asked for:

- §4 Statement of the Invention (broadest claim shape in prose)

- §5 Primary embodiment — 12 m rigid BEV, 1+6 doors, the bilateral 3+3 disposition explicitly called out as novel

- §6 Distinction from prior art — full table benchmarking against conventional buses, curbside BRT, central-median BRT, airport apron buses, light rail, informal goods-truck shuttling

- §7 Alternate embodiments — length, powertrain, floor-height, interface, actuation, and control variants

- §9 Claims — 20 in total: 3 independent (vehicle, platform, system) + 17 dependents, including a method claim for the operational sequence and a sunken-bay variant claim

- §10 Drawings list (8 figures specified for the complete specification filing)

- §12 Abstract, §13 declaration block with signature line

Next sensible step before filing: have a patent agent stress-test claim 1's breadth (the 900-1,300 mm floor-height range and the "3 + 3 platform-level doors" lower bound — those are the negotiable knobs) and commission the 8 figures listed in §10. The text itself is filing-ready for an Indian provisional.

Road-Wagon Bus Specification v1.0.docx

23 tables, 32 sections, ~51 KB. Drafted to AIS-052 (ARAI Code of Practice for Bus Body Design and Approval) — deliberately departs from MoUD Urban-Bus-Specifications-II so we can spec a true 1,100 mm high floor. Covers:

- Vehicle class (Type-I SDX, Medium Capacity), dimensions, BEV powertrain (300 kWh / 250 km range, CCS-2 depot + optional pantograph)

- The novel door layout: D1 = nearside stepped door (≤ 400 mm first step, 3 internal steps to floor); D2-D4 nearside + D5-D7 offside platform-level doors at 1,100 mm, each ≥ 1,200 mm clear width with powered gap-filler plates and traction-interlock

- Standee math per AIS-052 2.2.17 → 32 seated + 60 standees = 92 design capacity

- Accessibility (AIS-119 + 2.2.19), emergency egress (E1/E2/E3 + door overrides), brakes/steering, structural, telematics

- Platform interface in two variants: 1,100 mm raised platform and 1,100 mm sunken bus-bay for event deployments

- Appendix A — clause-by-clause AIS-052 compliance matrix; Appendix B — 13.5 m, articulated, CNG, and event variants

In creating Type-I DLX classified specification, what were the tradeoffs in choosing the doors D2-D7 as additional service doors vs emergency exits doors?

Claude responded: The real tension is a single clause: AIS-052 2.

The real tension is a single clause: AIS-052 2.2.1.1.1 says service doors "shall be situated on the side of the vehicle nearer to traffic" (i.e., nearside). Our 3 offside doors (D5-D7) sit in regulatory grey if classified as service doors — which is what made Option B (call them emergency exits) tempting. Here's why we still picked Option A:

What Option B fixes — sidesteps the nearside rule entirely. Emergency exits are governed by cl. 2.2.4, which doesn't restrict which side they sit on. So you avoid one ambiguity.

What Option B breaks:

- Cycle life. A service door is engineered and tested for ~1 million open-close cycles (100+ stops/day for 25 years). An emergency exit is rated for handfuls. Naming something "emergency" and then cycling it every 90 seconds is a homologation red flag and an insurance one — the hardware would be undertested for its actual duty.

- Placement rules. Cl. 2.2.4.1 requires emergency doors to be on the side opposite the service door, located in specific bus zones (front/middle/rear depending on where the service door is). Six emergency exits literally cannot satisfy that geometry.

- Public Information System (cl. 2.2.25). Passengers see "EMERGENCY EXIT" signage above a door that 50 of them walk through every stop. The signage stops meaning anything, and the on-board safety announcement contradicts visible behaviour.

What Option A costs us — only drafting work. AIS-052 is silent on additional offside service doors above the required minimum, and silence-as-permission is a defensible position. ARAI has accepted analogous reasoning for Janmarg and Rainbow BRT high-platform buses, where offside is the primary service-door side. Our argument is structurally easier: nearside has the mandatory door (D1), and D5-D7 are extras.

Analogy — it's the difference between calling a fire-exit a "fire-exit" (used twice in a building's lifetime) versus labelling a normal corridor door as a "fire-exit" just to dodge a corridor-width code. The door still swings, but the misclassification follows the building forever: insurance, audits, evacuations all get tangled.

Bonus — Option A also keeps the patent claims internally consistent. The claims call these "platform-level service doors", which reads honestly to a USPTO/EPO examiner. If the ARAI filing called them "emergency apertures" but the patent called them "service doors", an examiner will eventually notice the drafting inconsistency and pull on that thread.

A hybrid (D2-D4 as service, D5-D7 as emergency) doesn't actually help — it adds asymmetric hardware, dual operator HMIs, and still runs into cl. 2.2.4.1 placement rules for the three "emergency" exits.