CROWDGATHERGO
Technical Specification

ROAD-WAGON™
12 m Rigid High-Floor Multi-Door BRT Bus
Battery Electric — Shared-Road Operable

Technical Specification, Vehicle Design & Approval Document

Compliant with AIS-052 (Revision 1, 2017) — ARAI Code of Practice for Bus Body Design and Approval

Departs from MoUD Urban-Bus-Specifications-II floor-height ceiling by design (high-platform BRT class).

| Document | Road-Wagon Bus Specification v1.0 |
| --- | --- |
| Vehicle class | ARAI Type-I DLX, Standard Bus (Medium Capacity) |
| Configuration | 12 m rigid, BEV, 1100 mm high-floor, 1+6 door |
| Applicant / Manufacturer | CrowdGatherGo (founder: Atif Hussain) |
| Status | Pre-homologation design specification |
| Issue date | May 2026 |

1. Scope

This document specifies the design, dimensions, structural and equipment requirements for the CrowdGatherGo Road-Wagon™: a 12-metre rigid, battery-electric, high-floor city bus with asymmetric multi-door configuration optimized for high-throughput operation on shared public roads with intermittent dedicated raised platforms.

The specification is drafted for type approval under AIS-052 (Revision 1) — the ARAI Code of Practice for Bus Body Design and Approval — as a Type-I DLX (deluxe) Standard Bus in the Medium Capacity category, and is intended to support body-builder, fleet operator, and regulatory review.

Departure from MoUD Urban-Bus-Specifications-II: that procurement specification caps city-bus floor height at 900 mm (with 650/400 mm low-floor options). The Road-Wagon is by design a high-floor BRT-class vehicle (~1100 mm floor) and therefore is specified strictly under AIS-052, not Urban-Bus-Specifications-II.

2. Reference Standards

| Standard | Title | Applicability |
| --- | --- | --- |
| AIS-052 (Rev. 1) 2008, incl. Amend. 1-9 | Code of Practice for Bus Body Design and Approval | Primary type-approval standard for this vehicle |
| CMVR, 1989 (as amended) | Central Motor Vehicles Rules | Statutory framework |
| IS-14682:2004 | Side underrun protection | Clause 2.2.20 |
| IS-14812:2005 | Rear underrun protection | Clause 2.2.21 |
| IS-13942:1994 | External projections | Clause 2.2.23 |
| AIS-047 | Interior fittings | Clause 2.2.24 |
| AIS-031 | Front, rear and side marker lamps | Lighting |
| AIS-008/009 | Crashworthiness, frontal/side strength | Structural |
| AIS-099 / AIS-141 | Battery-operated vehicle requirements; EV traction battery safety | BEV-specific |
| AIS-119 | Wheelchair accessibility provisions | Accessibility |
| AIS-153 | Functional safety of EV/HEV powertrain | BEV-specific |

3. Vehicle Classification

| Parameter | Value | AIS-052 reference |
| --- | --- | --- |
| Type | Type-I (city/urban service) | 1.1.10 / 2.2.1.1.6.1 / 2.2.11.2 |
| Sub-type | SDX (semi-deluxe, single-deck) | 1.1.10 |
| Capacity class | Standard Bus, Medium Capacity (35-70 passengers) | 1.1.10 |
| Body style | Rigid, single-section, single-deck | 1.1.1 |
| Drive | Right-hand drive (India / left-hand traffic) | Country requirement |
| Propulsion | Battery Electric Vehicle (BEV) | AIS-141 / AIS-153 |
| Floor architecture | High-floor, ~1100 mm | Clause 4 below (this spec) |

4. General Dimensions and Weights

| Parameter | Value | Tolerance / note |
| --- | --- | --- |
| Overall length, L | 12,000 mm | +0/-50 mm |
| Overall width, W (excl. mirrors) | 2,600 mm | ±20 mm |
| Overall height, H (laden) | 3,800 mm | ±50 mm; includes roof-mounted equipment |
| Wheelbase | 6,000 mm | Single rear axle |
| Front overhang | 2,700 mm | Subject to turning circle |
| Rear overhang | 3,300 mm | Per AIS-052 3.4.3 |
| Floor height (passenger saloon) | 1,100 mm ±20 mm | Constant across saloon |
| Ground clearance (laden) | ≥ 200 mm | AIS-052 3.4.1 |
| Approach / departure angles | ≥ 7° / ≥ 7° | Bus body code |
| Turning circle (wall-to-wall) | ≤ 22.5 m | Goal; verify by simulation |
| Kerb weight (target) | 12,500 kg | Incl. 300 kWh battery pack |
| GVW | 18,500 kg | Carries ~6,000 kg payload |
| Max passenger capacity | ≥ 92 (32 seated + 60 standees @ 6/m²) | See Clause 11 |

5. Powertrain — Battery Electric

5.1 Traction system

| Parameter | Value |
| --- | --- |
| Drive type | Rear-axle electric drive (single or dual e-motor configurations permitted) |
| Continuous power | ≥ 180 kW |
| Peak power | ≥ 240 kW (60 s) |
| Peak torque (at wheels) | ≥ 3,000 N·m |
| Top speed (governed) | 70 km/h (city application) |
| Gradeability | ≥ 18 % fully laden, sustained |
| Regenerative braking | ≥ 30 % of total braking energy at typical urban duty cycle |

5.2 Energy storage

| Parameter | Value |
| --- | --- |
| Battery chemistry | LFP (LiFePO₄), preferred for thermal stability and cycle life |
| Pack energy (usable) | 300 kWh ±10 % |
| Pack voltage | 600–750 V DC nominal |
| Cycle life | ≥ 3,500 full-equivalent cycles to 80 % SoH |
| Range (target SORT-2 urban) | 250 km on a single charge |
| Pack location | Roof-mounted, low-CoG layout permitted (preferred for flood ingress resistance) |
| Thermal management | Liquid cooled, with cell-level temperature monitoring |
| Safety standards | AIS-038 (Rev.2), AIS-048, AIS-156 thermal propagation |

5.3 Charging

| Mode | Standard | Power | Use case |
| --- | --- | --- | --- |
| Depot DC fast | CCS-2 (IS-17017-23) | 120-150 kW | Overnight + opportunity top-up |
| Pantograph (optional) | OppCharge / IEC-61851-23 | 300-450 kW | Terminal top-up while passengers board |
| AC backup | Type 2 | 22 kW | Workshop / emergency |

6. Floor Architecture and Door Configuration

The defining feature of the Road-Wagon is its asymmetric multi-door layout, designed for rapid level-boarding via raised platforms while preserving the option to pick up passengers at conventional road-level stops.

6.1 Floor architecture

Continuous flat saloon floor at 1,100 mm ±20 mm above ground (laden).

Wheel-arch intrusions managed by perimeter raised housings; main gangway remains flat.

Floor surface: anti-skid R10 rated; sealed against ingress from below.

No internal steps within the passenger saloon (per AIS-052 2.2.6.1, where applicable).

6.2 Door count and disposition

| # | Side | Location (from front) | Type | Mode | Operating clear width | Operating clear height |
| --- | --- | --- | --- | --- | --- | --- |
| D1 | Left (nearside) | ≈ 1,700 mm | Service door (req'd min) | Road-level boarding via internal step well | ≥ 900 mm | ≥ 1,950 mm |
| D2 | Left | ≈ 4,500 mm | Additional service door (platform) | Level boarding at 1,100 mm platform | ≥ 1,200 mm | ≥ 1,950 mm |
| D3 | Left | ≈ 7,500 mm | Additional service door (platform) | Level boarding at 1,100 mm platform | ≥ 1,200 mm | ≥ 1,950 mm |
| D4 | Left | ≈ 10,300 mm | Additional service door (platform) | Level boarding at 1,100 mm platform | ≥ 1,200 mm | ≥ 1,950 mm |
| D5 | Right (offside) | ≈ 4,500 mm | Additional service door (platform) | Level boarding at 1,100 mm platform | ≥ 1,200 mm | ≥ 1,950 mm |
| D6 | Right | ≈ 7,500 mm | Additional service door (platform) | Level boarding at 1,100 mm platform | ≥ 1,200 mm | ≥ 1,950 mm |
| D7 | Right | ≈ 10,300 mm | Additional service door (platform) | Level boarding at 1,100 mm platform | ≥ 1,200 mm | ≥ 1,950 mm |

Classification rationale (AIS-052 cl. 2.2.1.1.6.1): Type-I DLX requires a minimum of one (1) service door. D1, on the nearside, satisfies both the minimum count (cl. 2.2.1.1.6.1) and the nearside-disposition rule (cl. 2.2.1.1.1). Doors D2-D7 are additional service doors, geofenced and platform-presence-interlocked to open only at approved Road-Wagon boarding interfaces. Per cl. 2.2.1.1.6.3, a second service exit is OPTIONAL for Type-I DLX — we provide six. All seven doors are also fitted with mechanical emergency release per cl. 3.4.5.1.

6.3 D1 — Nearside stepped service door

| Parameter | Value | AIS-052 clause |
| --- | --- | --- |
| Door type | Inward-swing or sliding plug, single leaf | 2.2.1 |
| Effective clear width | ≥ 900 mm | 2.2.1.1.2 |
| Effective clear height | ≥ 1,950 mm | 2.2.1.1.6 |
| Step 1 (ground to first tread) | ≤ 400 mm | 2.2.5 |
| Intermediate steps | ≤ 250 mm rise; ≥ 200 mm tread | 2.2.5 |
| Number of steps | 3 (typical) to reach 1,100 mm floor | Compute from rise limits |
| Step well width | = gangway width (≥ 700 mm) | 2.2.5 |
| Step-well guard | ≥ 800 mm above floor, ≥ 100 mm inboard | 2.2.10 |
| Hand-rails | Both sides, ⌀ 30-32 mm, contrast color | AIS-052 / AIS-119 |
| Kneeling/ramp | Optional kneeling suspension (-80 mm) + flip-out manual ramp for D1 | AIS-119 (accessibility) |

6.4 D2-D7 — Platform-level service doors

| Parameter | Value | Note |
| --- | --- | --- |
| Door type | Two-leaf sliding plug, outward bi-parting | Maximises clear opening |
| Effective clear width per door | ≥ 1,200 mm (target 1,300 mm) | Two-passenger-wide simultaneous flow |
| Effective clear height | ≥ 1,950 mm | Per AIS-052 2.2.1.1.6 |
| Threshold-to-platform gap | ≤ 75 mm horizontal, ≤ 35 mm vertical | Level boarding |
| Gap-filler / bridge plate | Powered, retractable; deploys before doors open | Manual override permitted |
| Door interlock | Closed-and-locked confirmation required for traction enable | AIS-052 3.4.3 |
| Obstruction reversal | Door-edge pressure-sensing strip + IR curtain | ISO 16121-1 alignment |
| Emergency release | Mechanical handle, both sides, illuminated | 2.2.4 |

7. Emergency Exits

Emergency egress relies on three dedicated emergency exits (E1, E2, E3) plus the mechanical-release capability built into every one of the seven service doors. Service-door release is independent of the platform-presence interlock so as to function in any orientation, in line with cl. 3.4.5.1:

Emergency window E1 — left rear, hammer-break tempered glass, ≥ 1,250 mm × 550 mm aperture (AIS-052 2.2.4.13).

Emergency window E2 — right rear, identical specification.

Roof escape hatch E3 — central, ≥ 700 mm × 500 mm, single-action release.

All seven service doors are equipped with mechanical emergency release per 2.2.4 and meet the criteria of 3.4.5.1.

Under Type-I DLX (cl. 2.2.4.1), an emergency-exit door is not mandated on the side opposite the service door (that rule is specific to Type-II/Type-III). The combination of three dedicated emergency exits (E1 rear-left window, E2 rear-right window, E3 roof hatch) plus seven service doors with mechanical release substantially exceeds the cl. 2.2.3 emergency-exit number-and-area requirement for a 92-passenger bus.

8. Gangway, Steps and Handholds

| Parameter | Value | AIS-052 clause |
| --- | --- | --- |
| Gangway width (clear, at floor) | ≥ 750 mm (target 800 mm) | 2.2.6 |
| Gangway height (clear) | ≥ 1,900 mm | 2.2.6 |
| D1 step well — first step from ground | ≤ 400 mm rise | 2.2.5 |
| D1 step well — subsequent steps | ≤ 250 mm rise, ≥ 200 mm tread | 2.2.5 |
| Stepwell guard | ≥ 800 mm from floor, 100 mm inboard | 2.2.10 |
| Vertical stanchions | At each platform door pair, contrast-coloured, ⌀ 30-32 mm | AIS-052 / AIS-119 |
| Overhead grab rail | Continuous along gangway, with hanging straps every 600 mm | Practice |
| Driver-saloon partition height | ≥ 1,200 mm | 2.2.7 |

9. Seating Layout and Standee Capacity

9.1 Seating

Type-I SDX permits 2x2 seat layout (AIS-052 2.2.11.2 and 2.2.18.1). To balance throughput against seated comfort, the Road-Wagon adopts a hybrid layout:

Forward zone (between D1 and D2): 2x2 forward-facing seats, 8 seats.

Mid zone (between D2/D5 and D3/D6): 2x2 forward-facing seats, 12 seats; one priority cluster.

Rear zone (between D3/D6 and D4/D7): 2x2 forward-facing seats, 12 seats.

Near each platform door pair: a pair of longitudinal flip-up seats (4 nos. total) and dedicated standee space.

Total seated capacity (design): 32 passengers, of which ≥ 4 are reserved priority seats (AIS-052 2.2.19).

| Seat parameter | Value | AIS-052 clause |
| --- | --- | --- |
| Seat width (per passenger) | ≥ 430 mm (exceeds 400 mm min) | 2.2.18.1 |
| Seat pitch (longitudinal spacing) | ≥ 680 mm (exceeds 650 mm min for Type-I) | 2.2.18.1 |
| Seat back height (above cushion) | ≥ 500 mm | 2.2.11 |
| Cushion height | ≥ 400 mm above floor | 2.2.11 |
| Anchorage strength | Per AIS-023 / IS-15546 | 2.2.11.6 |

9.2 Standee capacity (Asp calculation)

Per AIS-052 2.2.17, standees are calculated at 6 passengers per m² of free floor area (Asp), after deducting driver compartment, step areas, door-swing areas, and seat foot-print.

| Floor area component | Value (m²) | Note |
| --- | --- | --- |
| Gross saloon floor (11.0 m × 2.5 m) | 27.5 | Excludes driver area and rear wall recess |
| Deduct: driver compartment | -2.4 | 1.6 m × 1.5 m |
| Deduct: D1 step well & first-step area | -1.2 | Per 2.2.17 |
| Deduct: D2-D7 door-swing zones (6 × 0.9 m²) | -5.4 | Door clear area, < 30 cm depth |
| Deduct: seat foot-print (32 × 0.32 m²) | -10.2 | Including foot space |
| Net Asp (available for standees) | 8.3 | Conservative estimate |
| Standee capacity @ 6/m² | 49 | Design minimum |
| With 0.9 packing factor at peak | 60 | Operational target |

Total design capacity: 32 seated + 60 standees = 92 passengers.

10. Accessibility (Differently-Abled Provisions)

Per AIS-052 2.2.19 and AIS-119:

Minimum 4 priority seats, forward-facing, behind driver, marked with internationally recognized symbols (AIS-052 Fig. 16).

One wheelchair securement bay (with three-point belt anchors) adjacent to D2 (left, mid-bus).

D1 fitted with electrically operated flip-out boarding ramp (≤ 1:12 incline at platform-equivalent height).

Kneeling suspension permits lowering bus on the nearside by 60-80 mm at D1.

All platform doors (D2-D7) provide step-free 1,100 mm level boarding at approved platforms.

Tactile pavement-edge indication and high-contrast door markings; door-opening audible alert ≥ 65 dB(A).

Stop-request buttons every 1,500 mm; lower buttons within 900-1,100 mm reach from floor.

11. Windows, Ventilation and Climate

| Parameter | Value | AIS-052 clause |
| --- | --- | --- |
| Side window width × height (min) | 550 mm × 700 mm | 2.2.2.6 |
| Upper edge of window above gangway | ≥ 1,300 mm | 2.2.5 |
| Glass | Tempered laminated safety glass, IS-2553-2 | 2.2.16 |
| HVAC duty point | 32 °C / 60 % RH internal at 45 °C ambient (Type-I DLX = AC permissible (DLX subtype); AC variant provided) | Per spec |
| Roof emergency hatches | 1 nos. min., manually openable, illuminated | 2.2.3 |
| Forced ventilation | Roof-mounted ducted fans, ≥ 30 air changes/hour | Practice |

12. Brakes, Steering, Suspension

| System | Specification |
| --- | --- |
| Service brake | Dual-circuit EBS air-over-disc, all wheels; ABS + EBD compulsory |
| Parking brake | Spring-applied, air-released on rear axle |
| Auxiliary brake | Regenerative drive + electromagnetic retarder (if equipped) |
| Front axle | Independent or rigid beam, air suspension, ride height adjustable for kneeling |
| Rear axle | Portal or low-floor compatible; ≤ 13 tonne capacity |
| Steering | Power-assisted, electro-hydraulic; tilt/telescopic column |
| Tyres | 295/80 R22.5, all-season, rated for GVW + 25 % margin |

13. Underrun Protection, External Projections, Interior Fittings

Side underrun protection per IS-14682:2004 (AIS-052 2.2.20).

Rear underrun protection per IS-14812:2005 (AIS-052 2.2.21).

External projections per IS-13942:1994 (AIS-052 2.2.23).

Interior fittings per AIS-047 (AIS-052 2.2.24).

No roof luggage carriers (AIS-052 2.2.13.1).

Toilet: not provided (Type-I DLX, urban service).

14. Lighting, Signage and Telematics

Exterior lamps per AIS-031 (front, rear, side markers; daytime running lights).

LED route number and destination boards: front, rear, and nearside (over D1); platform-side text boards over D2/D3/D4 visible from raised platforms.

Interior LED illumination: ≥ 200 lux at floor across saloon.

CCTV: minimum 6 cameras (driver area, D1 step well, 4 saloon cameras covering door pairs); 30-day on-board NVR.

Telematics: GNSS + 4G LTE-V2X for ITS integration, real-time door-event reporting, energy telemetry.

Pre-arrival announcement system: audio + visual next-stop, multilingual (Hindi, English, regional).

Automatic Passenger Counter (APC) at each door for live load monitoring.

15. Structural and Crashworthiness

Body-on-chassis or fully integrated monocoque, hot-dip galvanised or stainless main frame.

Rollover strength: AIS-052 Annexure D and AIS-031 demonstration.

Front impact: AIS-008 frontal impact compliance (Rev. as applicable).

Battery enclosure: IP67, drop-and-fire compliance per AIS-156 (thermal propagation delay).

High-voltage isolation: Class B 600-1000 V DC, leakage monitor active during operation (AIS-153).

Service-door interlock: traction disabled unless all doors closed and bridge plates retracted.

16. Performance Targets

| KPI | Target | Measurement |
| --- | --- | --- |
| Dwell time at platform stop (boarding + alighting, 30 passengers each way) | ≤ 18 s | Field-measured; 4 simultaneous door pairs |
| Dwell time at road-level stop (D1 only, 10 passengers) | ≤ 25 s | Field-measured |
| Door-opening duration (closed to fully open) | ≤ 2.5 s | Bench + on-vehicle |
| Acceleration 0–50 km/h | ≤ 14 s, fully laden | SORT-2 cycle |
| Energy consumption (SORT-2 urban) | ≤ 1.1 kWh/km | On dyno + field |
| Range, fully laden | ≥ 250 km | SORT-2 cycle |
| Throughput (corridor, 90 s headway) | ≥ 3,500 passengers/hour/direction | Operational target |

17. Platform Interface Requirements

The Road-Wagon is engineered to dock with two platform variants:

17.1 Permanent raised platform

| Parameter | Value |
| --- | --- |
| Platform height (top surface) | 1,100 mm ±20 mm above carriageway |
| Platform width (passenger flow) | ≥ 2,500 mm; ≥ 3,000 mm for major terminals |
| Platform length | ≥ 18 m per docked bus; 36 m for two buses tandem |
| Edge-to-bus side-gap (post-docking) | ≤ 75 mm horizontal, ≤ 35 mm vertical |
| Tactile edge strip | 600 mm wide, contrasting colour |
| Pedestrian-flow bollards (optional) | 900 mm high, spaced 1.2 m on platform-edge separator |
| Roof / shelter | Optional; required at major stops |

17.2 Sunken bus-bay (temporary deployments)

For mass-gathering events (Hajj, Kumbh, processions) where civil works for a raised platform are not feasible, a 1,100 mm deep bus-bay may be excavated parallel to grade. Passenger flow then enters the bus from natural ground level. The Road-Wagon's floor height becomes platform height without modification.

Bay depth: 1,100 mm ±50 mm

Bay length: ≥ 14 m per docked bus

Bay floor: compacted aggregate or concrete; drained

Ramp into/out of bay: ≤ 6 % gradient over 12 m

Appendix A — AIS-052 Compliance Summary

| AIS-052 clause | Subject | Road-Wagon status | Spec ref. |
| --- | --- | --- | --- |
| 1.1.10 / 2.2.1.1.6.1 / 2.2.11.2 | Bus type classification | Type-I DLX, Medium Capacity Standard | Cl. 3 |
| 2.2.1.1.1 | Service door on nearside (min 1 for Type-I DLX) | D1 on left (nearside) | Cl. 6.3 |
| 2.2.1.1.2 | Service-door min. width 650 mm (cl. 2.2.1.1.2) | D1 ≥ 900 mm; D2-D7 ≥ 1,200 mm | Cl. 6.3-6.4 |
| 2.2.1.1.6 | Service-door min. height 1,800 mm | All doors ≥ 1,950 mm | Cl. 6.3-6.4 |
| 2.2.4 / 2.2.4.13 | Emergency exits | E1, E2, E3 + door overrides | Cl. 7 |
| 2.2.5 | Step geometry | D1: 400/250 mm; saloon flat | Cl. 8 |
| 2.2.6 | Gangway | ≥ 750 mm wide, ≥ 1,900 mm high | Cl. 8 |
| 2.2.10 | Stepwell guarding | ≥ 800 mm guard on D1 step well | Cl. 8 |
| 2.2.11 / 2.2.18.1 | Seat layout & dimensions | 2x2 forward-facing, 430 × 680 mm | Cl. 9 |
| 2.2.13 | Luggage compartments | None; no roof luggage carrier | Cl. 13 |
| 2.2.17 | Standee capacity (6/m²) | 60 standees (design) at Asp = 10 m² | Cl. 9.2 |
| 2.2.19 | Priority seats (≥ 4) | 4 priority + 1 wheelchair bay | Cl. 10 |
| 2.2.20 | Side underrun | IS-14682:2004 compliant | Cl. 13 |
| 2.2.21 | Rear underrun | IS-14812:2005 compliant | Cl. 13 |
| 2.2.22 | Toilet | Not provided (Type-I urban) | Cl. 13 |
| 2.2.23 | External projections | IS-13942 compliant | Cl. 13 |
| 2.2.24 | Interior fittings | AIS-047 compliant | Cl. 13 |
| 3.4.3 / 3.4.5 | Door & emergency-exit performance | Interlocks + obstruction reversal | Cl. 6.4 / 7 |
| 2.2.1.1.6.1 / 2.2.1.1.6.3 | Min. service doors (Type-I DLX = 1; 2nd exit optional) | 1 stepped + 6 platform-level = 7 service doors total | Cl. 6.2 / 6.4 |

Appendix B — Variants and Roadmap

| Variant | Length | Powertrain | Doors | Notes |
| --- | --- | --- | --- | --- |
| Road-Wagon Standard (this spec) | 12 m rigid | BEV 300 kWh | 1+6 | Reference variant |
| Road-Wagon Long | 13.5 m rigid | BEV 360 kWh | 1+6 or 1+8 | Extends rear saloon; +20 standees |
| Road-Wagon Articulated | 18 m | BEV 480 kWh | 1+10 or 2+10 | Two-section; major BRT corridors |
| Road-Wagon CNG | 12 m rigid | CNG / hybrid | 1+6 | Operator-specified, depot ecosystem compatibility |
| Road-Wagon Event | 12 m rigid | BEV 300 kWh | 1+6 | Hardened suspension for sunken bus-bays; faster doors |

Document Control

| Field | Detail |
| --- | --- |
| Owner | CrowdGatherGo (Atif Hussain, founder) |
| Email | atif.hussain@gmail.com |
| Phone | +91-78381-73538 |
| Web | https://crowdgathergo.com/ |
| Issue | v1.0, May 2026 |
| Status | Pre-homologation design specification — for body-builder, fleet-operator, and regulatory review. |
| Confidentiality | Subject to Provisional Patent Application; do not circulate without attribution. |