CrowdGatherGo

Move more people, without moving more steel.

PROPOSAL  ·  REVISED (v2)

Slot-Based Timetable Optimisation

Lifting trunk-corridor throughput toward ~500 train-slots/day on existing running lines — validated offline first

Prepared for

The Operations Leadership, South Central Railway

Office of the General Manager & Principal Chief Operations Manager

Rail Nilayam, Secunderabad

Prepared by

Atif Hussain — Founder, CrowdGatherGo

B.Tech (CSE), IIT Kanpur  ·  MBA, IIM Bangalore

atif.hussain@gmail.com  ·  +91 78381 73538  ·  crowdgathergo.com

9 June 2026  ·  Confidential

# Executive Summary

The problem. South Central Railway's core trunk sections — notably Balharshah-Kazipet and Kazipet-Wadi on the Delhi-Chennai Grand Trunk route — run above 100% line-capacity. Saturation caps freight loading, erodes punctuality, and pushes the zone toward doubling and tripling works that cost years and ~Rs 20 crore per kilometre.

The proposition. Run every through-train on the main line at the section's uniform non-stop speed, evenly spaced on a 5-minute headway, and absorb all stopping at elongated station loops rather than on the running line. Because the main line then carries only full-speed, evenly-spaced trains, it behaves like a homogeneous freeway — lifting a saturated double-line corridor toward ~500-576 train-slots per day, without new running lines and without waiting for a Dedicated Freight Corridor.

Adopt it in steps. The work decomposes into four components. Two are useful on their own — a seconds-accurate kilometre-post timetable, and a live seconds-deviation readout that helps drivers self-correct — and together they build up to the slot-based throughput gain (component 3). A fourth, the optional WhatsComing locomotive-tracking app, lets trains pack tighter still, with improved safety.

Proof already in hand. This is not a concept sketch. A second-accurate, 5-minute-slot timetable has already been built for the New Delhi-Chennai (NDLS-MAS) route, mixing four train classes, alongside a stop-physics model that quantifies the time each halt costs. We seek to validate that model on SCR's own section data.

Zero-risk path in. We lead with an offline simulation that must first reproduce SCR's current Master Chart (the 24-hour time-distance train graph) from the live Working Time Table (WTT) to within minutes. No train is touched until the operating department is satisfied. Phases 0-1 are offered pro bono.

Credentials. The developer is an alumnus of IIT Kanpur (B.Tech, Computer Science) and IIM Bangalore (MBA), with analytical capacity-unlocking work across both rail and road.

# 1. The Capacity Challenge

## 1.1 Saturated trunk routes

After the 1 June 2026 reorganisation, South Central Railway comprises the Secunderabad, Hyderabad and Nanded divisions. SCR identifies the Gudur-Balharshah and Kazipet-Wadi (via Secunderabad) corridors as heavily saturated; the Mudkhed-Dhone corridor has been cited near 167% line-capacity. On Kazipet-Balharshah, coal accounts for roughly 40% of goods traffic — a slow, heavy stream sharing rails with 130 km/h services.

## 1.2 Why conventional answers are slow

Track doubling runs about Rs 20 crore per kilometre over years; Automatic Block Signalling adds roughly 15-20% capacity at ~Rs 2.5 crore per route-kilometre. Both are needed, but neither extracts the capacity already latent in how the timetable is built — which is the gap this proposal targets, faster and cheaper.

## 1.3 The national stakes

Rail moves freight with 75-90% less energy than road, yet rail's freight share has slipped to about 27% against the National Rail Plan target of 45% by 2030. India imports nearly 89% of its crude at a net oil bill near $100 billion a year, dominated by transport. Throughput on saturated corridors is therefore an energy-security and forex lever, not merely an operating metric.

# 2. The Approach: a Freeway for Trains

## 2.1 Four building blocks — the first two are deployable on their own

The approach is built from four components. The first two each deliver value independently and can be adopted on their own; together they build up to the throughput prize in component 3. Component 4 is an optional safety layer.

| Component | What it is | Standalone value and role |
| --- | --- | --- |
| 1. Seconds-accurate km-post timetable [independent] | A timetable accurate to the second at every kilometre post, interpolated by physics-based spreadsheet formulae from non-stop trial runs. | On its own: a sharper Working Time Table and Master Chart, and a precise punctuality benchmark. Role: the measurement backbone every slot is built on. |
| 2. Live seconds-deviation feedback [independent] | A live readout to each loco crew of how many seconds they are ahead of or behind their timetable, so they apply corrective measures. | On its own: punctuality gains on today's network, no slotting needed. Role: keeps trains glued to their slots so tight headways hold. |
| 3. Slot-based 5-min-headway timetabling [the goal] | Uniform main-line speed; stops absorbed at elongated loops; each stop drops a slot. No Dedicated Freight Corridor. | The throughput prize that components 1 and 2 build up to. |
| 4. WhatsComing loco tracking [optional] | Optional locomotive-tracking / driver gap-awareness app to pack trains more tightly with improved safety. | Lets headways tighten further, safely. RDSO-cleared before any reliance. |

## 2.2 Pull stopping off the main line

The capacity of a double-line section is destroyed by two things: trains halting on the running line, and fast trains catching slow ones. We remove both from the main line. Every through-train holds the main line at the section's uniform non-stop speed, evenly spaced on a 5-minute headway. Any train that must stop peels onto an elongated station loop, halts there, and rejoins. The main line therefore only ever carries full-speed, equally-spaced trains — the condition under which ~500-576 slots per day becomes physically attainable.

## 2.3 How speed differences are absorbed — without a Freight Corridor

A stopping train brakes, takes the loop, dwells, and re-accelerates. Our stop-physics model puts the running-time penalty at roughly 80 seconds per stop (about 1.5 km to brake and 2.9 km to regain speed from ~90 km/h) plus its dwell. Having lost that time, the train simply surrenders its slot and takes a later one. The outcome is variable gaps under a uniform headway: trains differ in average door-to-door speed while every train on the main line runs at the same instantaneous speed. This needs neither a Dedicated Freight Corridor nor freight removed from the network — only a shared main-line running speed.

## 2.4 The honest precondition: freight homogenisation

For this to hold, every through-train must be able to make the section's non-stop speed while running. Under-powered freight is given added traction — long rakes can run on two contiguous slots with distributed power (engines roughly every 600 m) — or pathed in dedicated windows. The real residual limit is heavy-haul that is physically speed-restricted (loaded coal cannot reach trunk passenger speed regardless of power); the pilot's job is to quantify exactly how much traffic that affects and how to window it. We state this plainly rather than assume it away.

## 2.5 The enabling tools: slotting and WhatsComing

Train slotting. Survey each section's non-stop running time to the second and the kilometre-marker, build repeating 5-minute slots, and assign each train a slot it keeps while running and drops by one at every stop.

WhatsComing (optional locomotive tracking, low-cost). A RailTel-backed locomotive-tracking and gap-warning app over existing track-circuit and block logic. Once lane discipline keeps relative motion between consecutive trains low, gaps can be tightened safely. Any safety-bearing role would require independent RDSO assessment and operating-department clearance — presented for evaluation, not asserted as proven.

## 2.6 What we actually deliver

- A faithful section model built from SCR's Working Time Table (WTT), sectional running times (SRT), gradients, block sections and loop lengths.

- A reproduction proof — the model must reproduce SCR's current Master Chart (24-hour time-distance train graph) within minutes before proposing anything.

- A conflict-free path uplift — how many additional paths can be inserted into the Working Time Table under existing precedence and crossing rules, loop and maintenance-block constraints.

- A loop-and-overtake plan — where each train takes precedence or is crossed at a loop, drops its slot and rejoins, delivered as a revised Master Chart that self-stabilises against drift.

# 3. Evidence and Precedent

## 3.1 A working slot model already exists

A second-accurate 5-minute-slot timetable has been built for the NDLS-MAS Grand Trunk route, mixing four train classes on a double line:

| Train class | Section run time | Daily slots |
| --- | --- | --- |
| Non-stop / Goods (full speed) | 30 h 30 m | 72 |
| Duronto / Rajdhani | 31 h 40 m | 72 |
| Superfast / Express | 36 h 30 m | 48 |
| Local / Passenger | 49 h 20 m | 96 |
| Order-of-magnitude total | per direction | ~288 (x2 ≈ 576) |

The slot arithmetic rests on a line-capacity model (on the order of 40 coaches per minute at 60 km/h on a double-line section, distance-based — analogous to classical traffic-stream formulae adapted to rail). The km-marker-level data is large and is intended to be delivered digitally via a companion mobile application.

Figure 1 — A tightly-coupled 5-minute-slot timetable for the NDLS–MAS Grand Trunk route (New Delhi · Jhansi · Bhopal · Nagpur · Warangal · Vijayawada · Chennai), 24 hours horizontal. Steeper lines are full-speed through trains; the shallower bundles are stopping trains that step onto loops and pick later slots — the main line stays uniform.

## 3.2 Indian Railways has done this before

Western Railway's Surat-Vadodara programme is a close precedent: a 131 km high-density section, river-constrained against new lines, where all stations already had loops. WR upgraded freight speeds, moved to 4-aspect automatic signalling and computerised CTC, and extended loops into the block sections — cutting block lengths from 3-11 km toward under 6, then under 4 km. That is the same throughput-over-speed philosophy this proposal applies, already validated inside IR.

# 4. Preconditions and Honest Limits

This is presented with its costs visible, because a safety-first operating department will (rightly) look for them.

- Loop works. Stopping-station loops are elongated (~1.5 km) and turnout entry/exit speeds raised. This is targeted civil and signalling spend — far below doubling, but it is not nothing, and replaces any claim of zero infrastructure.

- Freight speed. Sub-speed freight must be re-powered or windowed; physically speed-limited heavy-haul is the binding residual and is quantified in the pilot.

- Signalling. Intermediate Block Sections and Automatic Block Signalling enable safe tailgating; WhatsComing (locomotive tracking) is a frugal complement, RDSO-cleared before any reliance.

- Robustness. At a 5-minute headway delays propagate quickly; the slot buffer and gap-warning must absorb perturbations — explicitly stress-tested in simulation, not assumed.

- Junctions and terminals. These still bound the corridor and are modelled, not wished away.

# 5. Scope of Work and Deliverables

Staged so the railway commits nothing irreversible until each prior stage proves itself. Phases advance only on the operating department's explicit sign-off. Every output is expressed in the operating department's own currency — a Working Time Table (WTT) and a Master Chart — so the timetable and control-office cells can scrutinise and adopt it directly.

| Phase | Activity | Deliverable |
| --- | --- | --- |
| Phase 0 | Data assembly & section model build (one pilot section) | Calibrated microsimulation that reproduces the section's current Master Chart from the live Working Time Table (WTT) within minutes |
| Phase 1 | Path-uplift + precedence/crossing (loop-overtake) analysis under existing safety rules | Draft revised Working Time Table & Master Chart with quantified extra paths; freight-windowing & precedence plan; report |
| Phase 2 | Tabletop review with control office & operating staff | Control-office-validated Master Chart & WTT scenario; agreed measures of success |
| Phase 3 | Supervised trial of a few off-peak slots | Before/after throughput & punctuality; scale-up recommendation |

Pilot section. We recommend Balharshah-Kazipet — a segment of the already-modelled NDLS-MAS route, single-division and single-zone — as the primary candidate, with Kazipet-Wadi as the alternate. Because both carry heavy coal, the pilot will explicitly study how much freight needs re-powering or windowing.

# 6. Indicative Timeline

Offline phases carry no operational risk and run in parallel with normal working.

| Milestone | Indicative duration | Dependency |
| --- | --- | --- |
| Data assembly | 2-4 weeks | WTT, section data, control charts |
| Model build & reproduction proof | 3-5 weeks | Phase 0 data |
| Slot-uplift report | 2-3 weeks | Validated model |
| Tabletop review | 1 week | Operating-dept availability |
| Supervised off-peak trial | On SCR sign-off | CRS/RDSO clearance as applicable |

# 7. Investment and Terms

CrowdGatherGo is a frugal, founder-led practice. We price for proof, not promises.

| Engagement | Basis | Fee |
| --- | --- | --- |
| Phase 0 - Section model & reproduction proof | Proof of value | Pro bono |
| Phase 1 - Slot-uplift analysis & report | Proof of value | Pro bono |
| Phase 2-3 - Tabletop & supervised trial support | Scoped on results | By agreement |

What we ask in return. Access, under a data-sharing understanding, to the section Working Time Table (WTT), sectional running times, block-section and loop particulars, and the control office's Master Charts and control charts; and a little operating-team time for one tabletop review. No procurement, no capital outlay, no commitment beyond the next phase.

# 8. Expected Benefits

- Throughput. Toward ~500 train-slots/day on a saturated section, unlocked in weeks of analysis rather than years of construction.

- Faster freight. A disciplined main line moves freight at sustained speed between windows, improving wagon turnaround and loading.

- Service quality. Variable gaps under a uniform headway cut the cascading delays mixed-speed traffic causes today, and improve ticket availability through more paths.

- Profitability & deferred capex. Each extra path is revenue on existing assets, and latent capacity can defer doubling works at ~Rs 20 crore per kilometre.

- Oil-import & forex savings. Freight and passengers held on electrified rail rather than diesel road directly reduce imported-fuel demand and forex outflow.

# 9. Addressing the Operating Department's Concerns

- Isn't the throughput physically impossible because of mixed speeds? The mix is removed from the main line and absorbed at loops; the running-time cost of each stop is quantified in our stop-physics model, the full slot timetable already exists for NDLS-MAS, and the approach mirrors WR's Surat-Vadodara programme.

- Why trust an outside model? A working, second-accurate model already exists and will first be made to reproduce your current Master Chart. Only then do we propose changes — offline, and pro bono.

- Will this risk live operations? No. Phases 0-1 are entirely offline; nothing moves until you sign off on a validated scenario.

- Who carries the slow-train delay? It is explicit by design — variable gaps mean each stopping train openly takes a later slot, a deliberate operating choice, never a hidden cost.

- What about loops, freight power and maintenance? Loop elongation, freight re-powering/windowing, turnout speeds, traffic and power blocks and terminal limits are modelled as constraints, and stated as preconditions in Section 4.

# 10. Next Steps

- A 30-minute meeting with the operations team to fix the pilot section and the measures of success.

- A short data-sharing understanding for the pilot section.

- Commencement of Phase 0 - the offline section model - at no cost to SCR.

# Acceptance

Agreed on behalf of the parties below:

| For South Central Railway | For CrowdGatherGo |
| --- | --- |
| Name: ____________________________ Designation: ______________________ Signature & date: __________________ | Name: Atif Hussain Designation: Founder, CrowdGatherGo Signature & date: __________________ |